| 2024 ASBESTOS REGISTER – JACKSON HOUSE (Building 24), Rockhampton Girls Grammar School Date of register: 06/06/2024 NOTE: A demolition or refurbishment survey should be carried out by a competent person before undertaking refurbishment or demolition work. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Workplace address: |  | Rockhampton Girls Grammar School – Denham Street, Rockhampton City QLD 4700 |  | Name and contact details of competent person: | Lawrence Lyons Asbestos Assessor Licence No. 2317266 Environics (QLD) Pty Ltd ABN 76 354 368 913 Tel. 0429 128 145 |  | Workplace owner / manager: | Kasey Mitchell Facilities and Services Manager Rockhampton Girls Grammar School Tel. 0427 414 819 |
| Areas not assessed: |  | Ceiling cavities located beyond inspection openings, utility pits, and elevated areas where safety measures cannot be practically implemented. |  |  |  |  |  |  |
| Asbestos related documents: |  | ENV-2023-062 Ver 1-4 Asbestos Management Plan & ACM Register - RGGS, Denham St The Range Q 4700.pdf - Appendix One ENV-2023-062-RGGS-ASBESTOS REGISTER-v1-02.10.2024. (this document) - Appendix Two RGGS-691-PRO-ASBESTOS-MANAGEMENT-PROCEDURE-v1-24.07.2024 - Appendix Three RGGS-692-PRO-ASBESTOS-INCIDENT-MANAGEMENT-PROCEDURE-v1-24.07.2024 - Appendix Four RGGS-720-GDL-ASBESTOS-MANAGEMENT-POSTER-v1-24.07.2024 |  |  |  |  |  |  |
| Consultation: |  | Attach minutes of relevant future meeting to this document. |  |  |  |  |  |  |
| Disclaimer and Limitations: |  | Inspections were limited to accessible areas without dismantling or removing objects, including vegetation, insulation, coverings, furnishings, and personal belongings. Areas such as inside walls, between floors, and behind stored items were not inspected. No destructive procedures were performed, and relevant legislation was followed. For further details on regulations, please refer to additional materials. Compliance and Liability: This register does not serve as a 'Certificate of Compliance' unless specified and does not warranty against future asbestos issues. The report does not confirm that all areas are asbestos-free. The competent person is not liable for undetectable asbestos requiring structural disruption. |  |  |  |  |  |  |
| Date | Sample No. | Type of material identified or assumed and amount (Cross-sectional areas are estimates – measure before quoting) | Friable or non-friable asbestos | Condition of asbestos | Specific location of identified or assumed asbestos | Notes |  |  |
| 09/04/2024 | S11 | Chrysotile >30m2 | Non-friable unless removed using an abrasive method of removal i.e. lift & grinding concrete | Fair | Interior: Vinyl floor tiles of the fire escape stairwell at the northern end of the building | Walls and ceiling sheeting in same area, no asbestos detected |  |  |
| 09/04/2024 | - | Presumed Chrysotile >30m2 | Non-friable | Fair | Exterior: Upper most level – Soffits | Unable to access due to working at height restriction. Material of same appearance tested at Luck Building and found to contain Chrysotile asbestos. |  |  |

.

Page 1 of 3

| 2024 ASBESTOS REGISTER – FLORENCE HOUSE, Rockhampton Girls Grammar School Date of register: 06/06/2024 NOTE: A demolition or refurbishment survey should be carried out by a competent person before undertaking refurbishment or demolition work. |  |  |  |  |
| --- | --- | --- | --- | --- |
| NON-ASBESTOS MATERIALS The following materials were tested and found not to contain asbestos: |  |  |  |  |
| Date | Sample No. | Result of Analysis | Location | Notes |
| 09/04/2024 | S04 | No Asbestos Detected | Exterior: Ground floor ceiling of open covered area adjacent to library on the western side of the building | COHLABS Certificate of Analysis No. 24-4489 Extensive modern renovations have occurred in this area |
| 09/04/2024 | S07 | No Asbestos Detected | Exterior: Compressed cement sheeting attached to uncover stairwell infrastructure | COHLABS Certificate of Analysis No. 24-4489 |
| 09/04/2024 | S08 | No Asbestos Detected | Interior: Wall sheeting within fire cabinet on level one, hallway west | COHLABS Certificate of Analysis No. 24-4489. |
| 09/04/2024 | S09 | No Asbestos Detected | Interior: Wall sheeting within fire cabinet on level one, hallway east | COHLABS Certificate of Analysis No. 24-4489. |
| 09/04/2024 | S10 | No Asbestos Detected | Interior: Wall sheeting storeroom on level one | COHLABS Certificate of Analysis No. 24-4489. Some Synthetic mineral fibres (SMF) detected in sample |
| 09/04/2024 | S12 | No Asbestos Detected | Interior: Wall sheeting within fire escape stairwell, at northern end of building | COHLABS Certificate of Analysis No. 24-4489. |
| 09/04/2024 | S13 | No Asbestos Detected | Interior: Ceiling sheeting within fire escape stairwell, at northern end of building | COHLABS Certificate of Analysis No. 24-4489. |

Page 2 of 3

Photo 1 - Soffit of Upper Level - Presumed Asbestos Based on Testing Elsewhere

Page 3 of 3